# Versa AWS instance Upgrade SOP

AWS Instance Type Change

| Version | 1.0 |
| --- | --- |
| AWS Regions | US (Baseline) ¦ EMEA ¦ LATAM |

# Contents

1. Purpose and Scope

2. Critical Governance Control: Templates vs CLI (Non-Negotiable)

3. Pre-Change Validation Checklist

4. Step-by-Step Procedure — Instance Type Change to cx8xlarge

5. Post-Change Validation Checklist

6. Failure Conditions and Escalation Triggers

7. Contacts and Escalation

# 1. Purpose and Scope

Purpose: Provide an execution-ready SOP for upgrading Versa instances in AWS from their current instance type to a larger or smaller instance type e.g cx4xlarge to cx8xlarge or vice versa. This SOP is designed for engineering execution across US, EMEA, and LATAM regions and ensures a consistent, validated process for all teams.

Applies to: Versa VOS devices running in AWS environments (MCBB scope and applicable regional environments).

Out of scope: Non-Versa devices; non-AWS environments; unrelated MCBB components.

# 2. Critical Governance Control: Templates vs CLI (Non-Negotiable)

Policy Statement (MANDATORY): All persistent configuration changes MUST be performed through Versa Director template updates and pushed centrally. Direct CLI configuration is PROHIBITED except during an ACTIVE INCIDENT requiring immediate service restoration.

Reason: The next template push will overwrite local (CLI) settings, causing silent regression and drift.

Incident Exception (ONLY during active incident): CLI actions may be used to restore service immediately. After stabilization, the same changes MUST be applied to Versa Director templates and re-pushed to affected device groups. Post-incident reconciliation is REQUIRED to eliminate drift and prevent regression.

# 3. Pre-Change Validation Checklist

Complete ALL items before starting the instance type change.

Confirm AWS console access with appropriate IAM permissions to stop/start EC2 instances and modify instance types

Obtain the Versa checklist and cross reference [link to be provided here] - do not proceed without it

Confirm maintenance window with your regional lead and notify stakeholders

Identify the target Versa instance(s) for your region in the AWS Console (EC2 -> Instances)

Record the current instance type for rollback reference

Confirm instance health: no active alarms, both status checks passing (2/2)

Confirm no active sessions or critical traffic is running through the instance

Take an AMI snapshot/backup of the instance before proceeding

Validate that Versa Director templates are current and no pending pushes are outstanding

Open a ticket with the cloud provider to have an engineer on standby

## 3.1 Reference Pre-Change Command Set

# Confirm Versa service health (device CLI)

show system details

show interfaces

Check BGP Neighbor Status

show routing bgp neighbors

show routing bgp summary

Check Advertised Prefixes (to a specific neighbor)
show routing bgp neighbors <neighbor-ip> advertised-routes

Check BGP RIB / Routing Table

show routing bgp routes

show routing bgp routes detail

Filter by VRF (if using VRFs — common in MCBB)
show routing bgp summary vrf <vrf-name>

show routing bgp neighbors vrf <vrf-name>

show routing bgp neighbors <neighbor-ip> advertised-routes vrf <vrf-name>

show routing bgp neighbors <neighbor-ip> received-routes vrf <vrf-name>

Check specific prefix
show routing bgp routes <prefix/mask>

# 4. Step-by-Step Procedure — Instance Type Change to cx8xlarge

## Phase A — Stop the Instance

Log into the AWS Console and navigate to EC2 → Instances.

Locate the target Versa instance for your region.

Select the instance → Actions → Instance State → Stop.

Wait for the instance state to show Stopped before proceeding.

Confirm that traffic is fully on the other Versa instance and no issues

## Phase B — Change Instance Type to required instance size.

With the instance in Stopped state, select it.

Click Actions → Instance Settings → Change Instance Type.

Select cx8xlarge from the instance type dropdown.

Click Apply to save the change.

Confirm the instance type now shows cx8xlarge in the instance details panel before starting.

## Phase C — Start the Instance and Validate

Click Actions → Instance State → Start.

Wait for the instance to reach Running state and pass both status checks (2/2).

Confirm the instance type shows cx8xlarge in the instance details.

Validate Versa application health — confirm services are up and traffic is passing.

Check CloudWatch metrics for CPU, memory, and network to confirm normal behavior.

## Phase D — Return-to-Service Gate

NOTE: Do NOT return a Versa node to service until ALL of the following are confirmed.

Instance type confirmed as cx8xlarge in AWS Console

Both EC2 status checks passing (2/2)

Versa interfaces present and operational

Traffic passing normally through the instance

Use the Post-Change Validation Checklist (Section 5) as the gating criteria.

# 5. Post-Change Validation Checklist

Complete ALL items before marking the change as complete.

Instance type confirmed as cx8xlarge in AWS Console

Both EC2 status checks passing (2/2)

Versa services confirmed UP (no service errors)

Interfaces present and operational after restart

Traffic passing normally through the instance

CloudWatch/monitoring tool metrics reviewed — no anomalies in CPU, memory, or network

No active alarms triggered post-change

Notify NOC team that changes is complete

Log the upgrade status in the change management system

Document any issues encountered and resolutions applied

## 5.1  Reference Post-Change Command Set

# Confirm Versa service health (device CLI)

show system details

show interfaces

Check BGP Neighbor Status

show routing bgp neighbors

show routing bgp summary

Check Advertised Prefixes (to a specific neighbor)
show routing bgp neighbors <neighbor-ip> advertised-routes

Check BGP RIB / Routing Table

show routing bgp routes

show routing bgp routes detail

Filter by VRF (if using VRFs — common in MCBB)
show routing bgp summary vrf <vrf-name>

show routing bgp neighbors vrf <vrf-name>

show routing bgp neighbors <neighbor-ip> advertised-routes vrf <vrf-name>

show routing bgp neighbors <neighbor-ip> received-routes vrf <vrf-name>

Check specific prefix
show routing bgp routes <prefix/mask>

# 6. Failure Conditions and Escalation Triggers

| Condition | Action |
| --- | --- |
| Instance fails to reach Running state after start | Stop immediately, revert instance type to original (noted in pre-change), start and validate, escalate to Butau / Shaima |
| Versa interfaces missing after instance start | Do NOT return to service; escalate immediately |
| Both EC2 status checks not passing after 10 minutes | Escalate to Core Team and NOC to bring in additional resource |
| Traffic not passing after change | Escalate immediately; initiate rollback if needed |
| Configuration drift detected after template push | Enforce template-only policy; remove local overrides |

## 6.1 Rollback Procedure

Stop the instance immediately.

Revert the instance type to the original value (noted in Pre-Change Checklist).

Start the instance and validate it returns to a healthy state.

Restore from the AMI snapshot taken in Pre-Change if needed.

Escalate to core team immediately.

# 7. Contacts and Escalation

| Role | Contact |
| --- | --- |
|  | Michael Wynston |
|  | Butau Shaima |
|  | Anton Kirichenko |